Public Company ORLEN Lietuva

IMPLEMENTATION OF SEALING SYSTEMS WITH AIR SAMPLING LOCATIONS FOR VOC EMISSION REDUCTION IN WASTE WATER TREATMENT SHOP AT PC ORLEN LIETUVA

SCOPE OF WORK

| Rev. | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Date |  |  |  |  |  |  |

Table of Contents

| 1 | GENERAL | 3 |
| --- | --- | --- |
| 2 | INTRODUCTION | 3 |
| 3 | DETAILED SCOPE OF WORK | 3 |
| 3.1 | Introduction | 3 |
| 3.2 | Equipment to be Sealed | 3 |
| 3.3 | Types of Sealing Systems | 4 |
| 3.4 | Considerations for Technology Selection | 4 |
| 3.5 | Air Sampling and Emission Control | 4 |
| 3.6 | Target | 5 |
| 4 | TIME SCHEDULE | 5 |
| 5 | ENQUIRIES | 5 |
| 6 | QUALIFICATION REQUIREMENTS | 5 |
| 7 | GENERAL PROVISIONS APPLICABLE TO PROCUREMENT PROCESS | 6 |

- GENERAL

Public Company ORLEN Lietuva (hereinafter OWNER or OL) is looking for competitive technical / commercial proposal in line with requirements provided in CONNECT system regarding improvement proposal for the waste water treatment unit in OL including minimizing emissions emitted in to the atmosphere and eliminating odor surrounding the unit. Perform an assessment waste water treatment shop by preparing a study according to the conditions described in the scope of work.

- INTRODUCTION

ORLEN Lietuva is an integrated downstream company in Lithuania, with its principal asset being the Crude Oil Refinery located in Juodeikiai village, Mažeikiai District Municipality, in the northwestern part of Lithuania. ORLEN S.A. is the single shareholder of PC ORLEN Lietuva, which operates the only refinery in the Baltic States. The refinery is a vital part of the region's energy infrastructure, providing a range of petroleum products and contributing significantly to the local and regional economy. Its strategic location and advanced technological capabilities ensure that ORLEN Lietuva remains a key player in the Baltic energy market.

The waste water treatment shop at ORLEN Lietuva plays a crucial role in maintaining the refinery's operational efficiency and environmental compliance. The plant comprises a mechanical treatment block and a biological treatment block, both essential for managing the complex wastewater generated by the refinery. The mechanical treatment block handles the removal of large impurities, sand, and sediment from the wastewater, directing these materials to either further treatment or appropriate disposal. Two main wastewater streams enter the mechanical treatment unit. In Oil refining plants one of the main contaminates are oil products which are collected/ processed mechanical treatment facilities. In the mechanical treatment facilities large contaminates such as sand and are removed. These pollutants, once captured, are returned to the oil refining process. In the mechanical treatment unit (MTU), oil sludge is also collected and processed for secondary recycling. The MTU employs advanced processes such as flotation, during which particles of oil and colloidal substances in the wastewater are removed with the help of air bubbles and chemical reagents, achieving a higher level of wastewater purification before the biological treatment process. After biological treatment, the purified wastewater is returned for reuse in production processes, and the excess purified wastewater is discharged into external water bodies in compliance with strict technological, Lithuanian, and European Union environmental requirements.

- DETAILED SCOPE OF WORK

  - INTRODUCTION. The Contractor’s Scope of Work shall include the implementation of specified equipment and instrumentation to minimize emissions of Volatile Organic Compounds (VOCs) in accordance with the ORLEN Lietuva’s Waste Water Treatment Shop’s Air Pollution Reduction Plan for the period of 2023-2030. The primary objective is to install sealing systems that significantly reduce VOC emissions into the atmosphere and to establish/install air sampling locations according to set requirements.

  - EQUIPMENT TO BE SEALED. The following equipment units in the Mechanical and Biological Treatment Section are to be covered:

- Equipment from the System 1 – 28 units

- Equipment from the System 2 - 11 units

- REQUIREMENTS FOR SEALING SYSTEMS. The sealing systems must adhere to the following criteria:

    - Designed to function in EX zones, with specific fixed EX zone analyzers;

    - Allow operators to manually control covered or otherwise isolated equipment, for example: including manual adjustment and inspection of valves/gates;

    - When implementing VOC reduction solutions, they must be integrated with currently existing automation systems. During the operation of VOC emission reduction solutions, it is mandatory to ensure the ability to perform technical maintenance on the existing automation systems;

    - The sealing systems must be designed in such a way that, when there is a need to perform efficient equipment maintenance, they allow it to be carried out promptly.

    - Corrosion-resistant, taking into account factors such as wastewater temperature, H₂S, alkalis, acids and other chemicals, sunlight, oil, and oil refining products.

  - TYPES OF SEALING SYSTEMS. Potential types of sealing systems to be considered include:

    - Sealing systems with a sampling point for emissions in the working environment, or without it, with vapor recovery;

    - Sealing systems with a sampling point for emissions in the working environment, or without it, with vapor diversion to the OL plant flare system for combustion;

    - Sealing systems with a sampling point for emissions in the working environment, or without it, with an air/vapor filtration system;

    - Other possible alternatives proposed by the Contractor for VOC emission reduction.

  - CONSIDERATIONS FOR TECHNOLOGY SELECTION. When selecting the sealing technology, the following factors must be taken into account:

    - Location is within EX zones, a possibility of EX zones forming in the locations is present;

    - Manual control of mechanical treatment equipment and maintenance requirements;

    - Integration with the existing MTU automation systems;

    - Ensuring the possibility of equipment maintenance and repair after implementing sealing solutions;

    - Corrosion-inducing factors such as high temperatures, H2S, alkalis, acids, and other chemicals;

    - Continuous evaporation of various organic and inorganic compounds from industrial wastewater, including significant amounts of benzene, toluene, xylene, NH₃, and H₂S in the vapors.

  - AIR SAMPLING AND EMISSION CONTROL.

- For the monitoring and control of emissions into the ambient air, it is necessary to install air sampling points. The proposed systems must ensure representative sampling (i.e., measurement locations installed according to current air sampling requirements). This is a mandatory condition for VOC emissions monitoring and ensuring compliance with environmental standards as stated in the 2022 june 20 d. LR Ministry of Environmental Protection order Nr. D1-68 “Dėl stacionarių taršos šaltinių išmetamų į aplinkos orą teršalų ir teršalų aplinkos ore ėminių laboratoriniams tyrimams atlikti ėmimo, matavimų ir tyrimų atlikimo taisyklių patvirtinimo“.

  - TARGET. The primary target of this Scope of Work is to significantly reduce VOC emissions from the mechanical treatment facilities into the environment by installing technical measures in accordance with the "ORLEN Lietuva" Air Pollution Reduction Plan for the Treatment Facilities Department for the years 2023-2030. The Contractor's proposed VOC emission reduction solutions must correspondingly reduce the dispersion of odors. The Contractor must:

- Implement sealing systems that minimize VOC emissions;

- Install and maintain air sampling points for emissions control;

- Ensure that all equipment is coated with materials resistant to the aggressive effects of wastewater evaporation and other physical factors;

- Connect all sealed equipment to a closed air/vapor/odor collection system, directing them to a gas treatment unit or absorption system for hydrocarbon vapor recovery, if such a sealing solution is selected;

- Contribute to compliance with odor management and control requirements according to BAT 18 and 6;

- Ensure that the pollutant removal and/or collection efficiency and odor removal efficiency achieve 90-99% of the previously assessed pollution level.

- TIME SCHEDULE

  - Contractor shall provide a detailed work and implementation schedule.

- ENQUIRIES.

  - It is recommended that a potential contractor visit ”ORLEN Lietuva” waste water treatment shop to assess the current situation.

- QUALIFICATION REQUIREMENTS

  - The Contractor shall submit a list of design, dismantling, and installation works of equipment executed in the oil refining/chemical industry or industrial wastewater treatment plants over the last 5 years, along with references to other projects confirming the Contractor’s qualifications (which correspond to the implementation of tasks specified in the scope of work).

  - The Contractor shall submit design evaluations, certifications, and documentation proving the reduction of VOC emissions achieved through the implemented measures, demonstrating effectiveness via a specific quantitative value (e.g., tons per year, t/year) or by other qualitative metrics.

  - If construction works are to be performed, the Contractor or their subcontractors shall possess and submit to the client a valid certificate issued by “Valstybinės energetikos inspekcija (VEI)” for performing repair works on oil refining equipment.

  - The Contractor or their subcontractor shall confirm that the proposed technological solutions will include built-in air sampling points, which comply with the legal requirements stipulated in the appendix of the scope of work document.

- GENERAL PROVISIONS APPLICABLE TO PROCUREMENT PROCESS

  - Please provide your questions, if any, through ‘Questions/Answers’ in CONNECT. All explanatory material will be provided either via CONNECT or sent by email where it is not possible to do it via CONNECT;

  - The Proposal shall consist of two parts - Technical and Commercial in accordance with requirements provided in CONNECT system;

  - Technical Part of the Proposal shall include the following:

    - Reference list;

    - Works execution plan and schedule;

  - Please submit your Technical - Commercial Proposal via CONNECT system;

  - The Proposal shall be valid for at least six months after its receipt date;

  - All costs related to preparation of the Proposal shall be at your sole expense;

  - OWNER can extend the deadline for the submission of proposals upon receipt of Bidder’s motivated request. In case the OWNER agrees to extend the deadline, all potential Contractors giving notice of intention to participate in the tender will be notified about the new deadline;

  - By submitting a proposal, the Bidder confirms that all information provided by it is accurate and correct. Bidder has not concealed any circumstances that the OWNER should be aware of;

  - After submission of proposal the OWNER will not accept any changes to the commercial part (price, estimates) reasoned by the Bidder has not understood and/or incorrectly evaluated the scope, specifics of required works, materials, labor, etc.;

  - If proposal does not meet the tender requirements, the OWNER reserves the right to reject it;

  - OWNER shall have the right to revoke the Request for Proposal and reject/suspend the procurement process without indicating the reason thereof. Bidders shall agree not to file any claims against OWNER regarding the above provisions;

  - Bidders shall keep any and all information related to this RFP confidential. Bidders shall not disclose any information related to submission of the Proposals, negotiations and Contract signing without a written consent from OWNER for the disclosing of such information;

  - Any information and data shall be considered confidential, and shall be protected against disclosure directly or indirectly to any third party without a written consent thereto from Public Company ORLEN Lietuva. This information and data is for the attention of the indicated addressee only. If you are not the indicated addressee, you are not authorized to copy, disclose or distribute this information or data to other persons. In this case, please notify Public Company ORLEN Lietuva thereof by email or call the sending person, and delete all the received information and data.