Appendix no. 6

to the Guidelines for the location of Temporary Backup Facilities

for Contractors on the premises of ANWIL S.A.

# DETAILED SAFETY CONDITIONS

FOR TEMPORARY BACKUP FACILITIES ON THE PREMISES OF ANWIL S.A.

- General safety conditions for Temporary Backup Facilities.

  - Temporary Backup Facilities should be arranged at a minimum distance of  8 m from buildings, provided that the Temporary Backup Facilities may not be located less than 30 m from active production installations.

  - The Temporary Backup Facilities must be technically sound and comply with all applicable legal provisions, in particular ones related to technical and construction regulations, occupational  safety and health regulations, and fire safety and environmental protection regulations.

  - The Temporary Backup Facilities must be sited in such a manner that they do not obstruct the visibility of motorists and do not impede access to fire-fighting equipment (e.g. hydrants, evacuation assembly points, fire extinguishing and safety system activation points, etc.).

  - The Temporary Backup Facilities and their respective constituent objects, e.g. electrical equipment, should come with an up-to-date test of the effectiveness of electric shock protection and insulation resistance.

  - The power of the electrical equipment installed in the Temporary Backup Facilities must not exceed the maximum permitted power consumption with which the electrical installation was designed and constructed.

  - If the Temporary Backup Facilities serve both a social and an assembly function, they must be visibly divided into two parts serving, respectively, the social and assembly functions.

  - If the surroundings, including landscaping, so permit and the facilities comprising the Temporary Backup Facilities are suitable for this purpose, they may be founded on two levels.

  - It is not permitted to locate the Temporary Backup Facilities in hazardous areas or under trestle bridges.

# Occupational safety and health

  - Due to the location of the Temporary Backup Facilities within the premises of ANWIL S.A., the Contractor is required:

    - To obtain an opinion from the Maintenance Services unit responsible for the area in which such facilities are to be located on the need to equip the indicated Temporary Backup Facilities with a lightning protection system;

    - To identify potential significant hazards and risks to people and property that may emerge during construction, implementation, installation and commissioning, testing, operation  and decommissioning of the facilities comprising  or located at the Temporary Backup Facilities, and take effective measures to prevent such hazards;

    - To exercise extreme caution in all activities that may cause dangerous situations to occur and interrupt the operation of installations belonging to ANWIL S.A. or third parties;

    - To perform all works on the Temporary Backup Facilities, in particular assembly, in such a manner as to take account of the requisite safety and minimise the risk of loss of life or limb during construction, implementation, assembly  and  commissioning, testing, operation  and  decommissioning of   the facilities comprising or located on the  Temporary Backup Facilities.

  - The Temporary Backup Facilities should be equipped with sufficient first aid facilities, including first aid kits with first aid manuals, an inventory of the medical supplies in such first aid kits, a list of persons trained to administer first aid and a list of emergency telephones.

  - If necessary to store machines, devices or materials in the open air on the Temporary Backup Facilities, the Contractor is obliged to designate and visibly mark the storage areas.

# Fire protection.

  - At the Temporary Backup Facilities, the Contractor is responsible for carrying out its fire protection duties, including the maintenance of operational firefighting equipment to the extent necessary.

  - Work which are hazardous in terms of fire, on the premises of the Temporary Backup Facilities, may be performed having established the safety conditions based on relevant permits, in accordance with the principles on the performance of works on the premises of ANWIL S.A., described in separate IOR on the performance of particularly hazardous works.

  - The Temporary Backup Facilities must be equipped with handheld fire-fighting equipment, i.e. no less than

1 unit of firefighting equipment (6 kg of powder or 5 kg of CO2) for each  facility includedin the Temporary Backup Facilities.

  - It is forbidden to perform such activities on the premises of the Temporary Backup Facilities and in their immediate surroundings as may cause a fire, spread or impede evacuation or rescue and firefighting operations.

  - In need to refuel vehicles at the Temporary Backup Facilities, a suitable place must be designated and separated, in agreement with the Company Fire Department operating at ANWIL S.A.

# Environmental protection.

The following are categorically prohibited in and around the Temporary Backup Facilities:

- creation of dumping and storage  sites for unsorted waste related to the Contractor's operations;

- cutting trees and shrubs without the necessary prior consent from ANWIL S.A.;

- performance of activities causing damage to green spaces;

- using chemicals, i.e. substances and mixtures posing a risk to health, life or the environment without having up-to-date safety data sheets for such chemicals.