- General.

Maintenance of the storage tank Rz-45 in the Feedstock and Products Storage Shop of Operations Subdivision No 3 in 2026.

Rz-45, H-17.92 m, D-39.90 m, volume 20 000m3

Product: Diesel

Operating temperature: 50 ºC

2. Attachments.

2.1. Attachment 1. Design 704-1-70 book I.

2.2. Attachment 2. Photos.

3. Description and specifics of work.

3.1. General requirements.

3.1.1. It is required hereunder to repair and paint storage access platforms, foam generators of tank RZ-45. It is required to paint Rz-45 with anti-corrosive paint.

3.2. Work description. 3.2.1. Repairing the access platforms up to the requirements of LST EN ISO 14122-3: - Increasing the height of the internal guardrails of the access platforms on the roof to at least 1100 mm; the spacing between the guardrail beams shall be not more than 500mm, etc. - Repairing the access stairs and guardrails on the tank roof. - Installing safety gates (12 pcs.) for the access platforms of the foam generators. (Gates to be provided by the Contractor). - Repairing the stairs with safety cage and access platforms of the foam generators. - Repairing the access platforms of inlet valves of the tank and stair guardrails, etc. - Dismantling, repairing and painting the foam generators (6 pcs.). Installation work shall be commenced only after having it agreed with the mechanical engineer or responsible employee. - For additional information see Attachment. - The exact scope of repairs to the access platforms shall be assessed during the visual inspection of the tank. 3.2.2 Painting of Rz-45: - The external wall of the tank shall be painted fully from the top to the bottom, including the stairs, access platforms, and existing pipework near the tank. - The roof (exterior) of the tank shall be painted fully, including the access platforms and other roof-mounted equipment (nozzles, covers, etc.). - After the painting of the tank, the Contractor shall restore inscriptions on the wall and the roof. The contents and size of the inscriptions shall be agreed with the head of the unit and shall comply with the tank labeling requirements. - Access platforms with stairs of the inlet valves. - Foam generators (6 pcs.).

3.3. Special requirements (if any).

3.3.1. Contractor shall visit the worksite and get familiar with the actual scope of work. 3.3.2 Prior to work commencement the Contractor shall agree with OL and get approved the equipment repair and quality control plan. 3.3.3 Final documentation shall be provided within one month after the completion of works. The requirements applicable to repair documentation are defined in Procedure BM-6. 3.3.4 CONTRACTOR’S proposal shall include all the works described in the Scope of Work. Any minor work not included in this SOW though required for ensuring the functionality and completeness of the works shall be considered included in the proposal. 3.3.5 In the event of any conflicts between different documents, or in case technical information is not fully presented in the Scope of Work and Attachments, the CONTRACTOR shall be obliged to notify the OWNER or its authorized person thereof and follow his instructions. 3.3.6 Quality requirements applicable to painting: - All works shall be performed in strict compliance with the requirements set forth in normative acts of the Republic of Lithuania. - The Contractor must provide five-years warranty for all the works performed by it. Maximum corrosion rate after 5 years – Ri3 (ISO 4628-3). Warranty for hidden works shall be 10 years. Durability of coating - 15 years at least. Remedy of the works of non-compliant quality shall be at the Contractor's expense. - The Contractor shall be liable for all and any actions that affect the quality. The Contractor shall keep its work site clean, tidy and safe at all times. The Contractor shall remove from the work site all construction waste and debris generated in the course of the works performed by the Contractor, taking such waste to the Owner designated location in the territory of Public Company ORLEN Lietuva. - Upon completion of the works, the Contractor shall be required to remove its materials, equipment, etc. from the Owner's territory leaving the Owner's territory clean, safe and ready for use.

3.4. Requirements for work execution and documentation.

3.4.1 A work method statement shall be developed according to OL requirements for all maintenance works that require welding and shall be agreed with OL responsible specialists. 3.4.2 The Contractor shall develop repair documentation according to OL requirements for all completed repair works. The agreed and approved documentation shall be submitted to OL. 3.4.3 All disassembled and assembled flanged connections shall have an attached information plate (tag) indicating the name of the Contractor having performed the works, gasket type, ID of metalworker (from the submitted list of metalworkers) having assembled the flange, and the date. 3.4.4 Painting systems shall be selected by Contractor according to effective standards and shall be compatible with the fluid and environment and agreed with OL. 3.4.5 For painting work, the Contractor shall deliver the relevant work completion report with indication of the completed works, applied work methods, certificates of the used materials, hidden works, process control reports, etc.

3.5. Qualification and Technical Requirements for Contractor.

3.5.1 Valid certificate issued by the National Energy Regulatory Council (NERC) of the Republic of Lithuania authorizing maintenance/operation of crude processing facilities: Repair of crude oil processing units irrespective of their parameters.

3.5.2 List of subcontractors (if any). Where subcontractors are indicated, it shall be required to present their valid (NERC) certificates for respective works. If subcontractors are indicated, however not named and/or their valid NERC certificates are not provided, the proposal will be rejected or, in case of necessity, subjected to additional revision.

3.5.3 Company employees with employment contracts and rigger qualification.

3.5.4 Number of employees with employment contracts and crane operation manager (supervisor) certificates.

3.5.5 Company employees holding employment contracts and qualified as tank maintenance foreman

3.5.6 A list of company employees holding employment contracts and European Welding Engineer or European Welding Technologist (IWE, EWE or IWT, EWT) qualification certificates.

3.5.7 List of company’ welders and information on their qualifications, documents confirming welding qualification in accordance with EN287 or EN287+PED 97/23EC, welding processes 141 and 111, for steel group 1, for pipes with diameter from 20 mm and ≥ 25 mm, wall thickness from 3 mm to 24 mm.

3.5.8 Qualification requirements for painting work:

- A copy of the company registration certificate;

- A license (permit) issued by the Ministry of Environment or similar document of another country recognized in the Republic of Lithuania confirming the right to perform such construction works (Technical Construction Regulation STR 1.02.07:2004 http://www3.lrs.lt/pls/inter2/dokpaieska.showdoc_l?p_id=243876).

- A qualification certificate of construction manager for execution of exceptional significance construction works or similar document of another country recognized in the Republic of Lithuania confirming the right to perform such construction works (Technical Regulations for Construction STR 1.02.07:2004 http://www3.lrs.lt/pls/inter2/dokpaieska.showdoc_l?p_id=243876 ). A list of certified construction work managers.

4. Materials, equipment, and services to be provided by the Owner.

4.1. Insulation – another OL contractor;

4.2. Non-destructive testing – another OL contractor;

4.3. Blinding/isolation and complete preparation of Rz-45 for hot works – another OL contractor;

5. Materials, equipment, and services provided by Contractor.

5.1. Washing required for the preparation and painting of the tank – by Contractor. 5.2. Materials for the repair of access platforms and foam generators and for fabrication of guardrails – by Contractor. 5.3. Cranes, forklifts, lifting equipment and other special machinery needed for the works – by Contractor; 5.4. Additional materials for installation/repair works/painting, protection against rain, wind – by Contractor; 5.5. Scaffolding for interior and exterior painting works – by Contractor. 5.6. Temporary supports and supporting structures, their fabrication, installation and materials (if any) – by Contractor; 5.7. Concreting (foundations) and earthwork as well as materials if any required for the installation of access platforms – by Contractor. 5.8. Painting materials and equipment: - Contractor shall furnish the tools and equipment as well as transport, anti-corrosion paint, cover against wind and rain, etc. necessary for works. - Contractor shall maintain the temperature inside the tank required for painting. Steam will be supplied by the Owner. Contractor shall provide required additional heating equipment. - Contractor shall remove and/or dispose of the sandblasting waste. - Owner will provide connection points and electricity.

6. Requirements for work completion.

| 6.1. Work shall be deemed completed after performing all works indicated in the scope of work, conducting all required tests (with acceptable results) and submitting the respective documentation. |  |
| --- | --- |
| 6.2. Before the final completion of the works, the Contractor shall return to ORLEN Lietuva warehouses all unused materials, sort and deliver all resulting waste to OL waste management sites. 6.3. The Contract shall clean up the worksite before its transfer to the Owner. The transfer shall be executed by signing a site cleanup statement. |  |

7. Requirements for work acceptance.

7.1. Preparing and presenting repair documentation as prescribed by OL Procedure BM-6;

8. Requirements for work schedule.

8.1. Works shall be performed based on a schedule agreed with OL. Works shall be completed as soon as possible. Work duration (schedule provided by Contractor) is one of the main criteria for selecting a contractor. 8.2. Works shall be performed in one shift of 11 hours, 7 days a week. 8.3. The Contractor shall prepare a work schedule and submit it to OL for approval. 8.4. Preliminary start date – 7 June 2026; end date – 11 August 2026.

Other information required for procurement (information provided in this Item will not be sent to contractors).

9. The price in the proposal shall be broken down as follows: 9.1 Machinery; 9.2. Repair of access platforms and guardrails, materials. 9.3. Preparation and painting of access platforms and guardrails. 9.3. Scaffolds. 9.4. Foam generator demounting/installation, repair, painting. 9.5. Preparation and painting the wall (exterior), including scaffolding. 9.6 Preparation and painting the roof (exterior). 9.7. Hourly rates for 1x5x8; 1x5x11; 2x5x11; 1x7x8; 1x7x11; 2x7x11 schedules for: - Work manager - Welder - Pipe fitter - Watchman