- General data:

Main crude oil pipeline Mažeikiai – Būtingė:

Pipeline diameter – 22 inches;

Length in the territory of the Republic of Lithuania – 92 km;

Transported product – crude oil;

Operation pressure – 100 bar.

Offshore pipeline, on-shore section:

Pipeline diameter – 36 inches;

Length in the territory of the Republic of Lithuania – 3 km;

Transported product – crude oil;

Operation pressure – 12 bar.

Main product pipeline Ilukste – Biržai – Ventspils:

Pipeline diameter – 530 mm;

Length in the territory of the Republic of Lithuania – 87 km;

Transported product – diesel fuel;

The pipeline stands idle, product pumped out. Filled with nitrogen gas, pressure – up to 3 bar. Low danger to environment.

Main crude oil pipeline Polotsk – Biržai – Ventspils:

Pipeline diameter – 720 mm;

Length in the territory of the Republic of Lithuania – 87.384 km;

Transported product – crude oil, operation pressure – 39 bar.

The pipeline stands idle, crude oil pumped out, pipeline washed and mothballed. Filled with nitrogen gas, pressure – up to 3 bar. No potential danger to environment.

Main crude oil pipeline Polotsk – Biržai – Mažeikiai:

Pipeline diameter – 720 mm;

Length in the territory of the Republic of Lithuania – 225.514 km;

Transported product – crude oil, operation pressure – 51 bar.

The pipeline stands idle, crude oil pumped out, pipeline washed and mothballed. Filled with nitrogen gas, pressure – up to 3 bar. No potential danger to environment.

- Attachments

Attachment 1. Quantities and delivery terms of obligatory emergency response means and qualified staff.

Attachment 2. Unit rates and hourly rates for emergency response means, services and qualified staff.

- Attachment 3. Means to have at Būtingė terminal always.

- Description and Specifics of Services.

Public Company ORLEN Lietuva Pipeline and Terminal Operations Subdivision. Emergency response works for main crude oil pipelines Polotsk – Biržai – Ventspils, Polotsk – Biržai – Mažeikiai, Mažeikiai – Būtingė; main product pipeline Ilukstė – Biržai – Ventspils; on-shore section of offshore pipeline (from Terminal to the sea). Level I fieldwork: pollution containment, collection of pollutants from the surface of water and those penetrated into soil – up to 30 days. Continuation of works acc. to the Emergency Response Plan (Level II), if required.

  - In case of an emergency on main pipeline and receipt by the Contractor of a notification thereof by phone from the Owner’s authorized representative, the Contractor shall immediately confirm the receipt of such notification by SMS (to the mobile phone number specified on the list of responsible persons) and/or e-mail (to the address specified on the list of responsible persons). The Contractor shall make sure the Contractor’s representative is reachable on phone (by the number indicated in advance) 24 hours a day. In case of any changes to the phone number of the Contractor's representative or any circumstances, for which the representative may be unreachable, the Contractor shall immediately notify the Owner thereof and provide a phone number of another representative for notifying about an emergency.

  - In case of an emergency on main pipeline and receipt by the Contractor of a notification thereof by phone from the Owner’s authorized representative, the Contractor shall immediately, however within maximum 2 hours on working days or within maximum 4 hours in any other time, on weekends and public holidays, arrive at the site indicated by the Owner with qualified staff, primary emergency response machinery, equipment and materials (hereinafter – the means, see Attachment 1), and commence emergency response works. Since these are emergency works, the Owner and the Contractor agree that Work Orders for such may be prepared and signed by both Parties after actual commencement of works, but not later than on the next working day.

  - Normative documents, standards and other regulations to be observed by the Contractor for containment of and response to emergencies:

    - Rules for Operation of Main and Offshore Crude and Petroleum Product Pipelines approved by the Minister of Economy of the Republic of Lithuania with Order No. 4-207 dated 25 May 2007;

    - Rules for Protection of Main Crude Oil Pipelines approved by the Minister of Economy of the Republic of Lithuania with Order No. 1-131 dated 30 April 2010;

    - Law on Environmental Protection of the Republic of Lithuania (21 January 1992 No I-2223 (as amended));

    - 17 November 2009 Order No D1-694 by the Minister of Environment of the Republic of Lithuania regarding approval of Lithuanian environmental normative document LAND 9-2009 - Environmental Requirements for Remediation of Territories Contaminated with Petroleum Products;

    - 16 May 2006 Order No D1-228 by the Minister of Environment of the Republic of Lithuania regarding approval of the Procedure for Selection of and Preliminary Approval of Remedial Measures (Official Gazette, 2006, No 59-2099, as amended);

    - 30 April 2008 Order No D1-230 by the Minister of Environment of the Republic of Lithuania regarding approval of environmental requirements for remediation of territories contaminated with chemical substances (as amended);

    - 17 June 2008 Order No 1-104 by Director of Lithuanian Geology Survey under the Ministry of Environment regarding approval of Eco-Geological Investigation Regulations (Official Gazette, 2008, No 71-2759, as amended);

    - 9 September 2002 Order No 471 by the Minister of Environment of the Republic of Lithuania regarding approval of Methodology for Calculating Environmental Damage Compensation (Official Gazette, 2002 No 93-4026, as amended);

    - If the Contractor is going to perform waste management operations, it must meet all the requirements applicable to waste management companies prescribed by the Law on Waste Management of the Republic of Lithuania (Official Gazette, 1998, No 61-1726, as amended) and observe Waste Management Rules approved by the Minister of Environment with the 14 July 1999 Order No 217 (Official Gazette, 1999, No 63-2065, as amended), and the requirements established in Attachment 5.

    - In case of shipment of waste from LR to EU, the Contractor shall comply with the requirements of transboundary movement of waste, as established in Regulation (EC) No 1013/2006 of the European Parliament and of the Council of 14 June 2006 on shipments of waste, and comply with Order No D1-384 of 4 July 2007 by the Minister of Environment regarding amendment of Order No D1-207 as of 27 April 2004 by the Minister of Environment of the Republic of Lithuania regarding approval of Waste Shipment Procedure (Official Gazette, 2007, No 76-3036).

NOTE: The Contractor shall confirm that it is familiar with the above-mentioned legislation and other applicable standards and requirements.

- Materials, equipment, and services to be provided by the Owner.

  - Marking of underground utility lines (cables, piping);

  - Supervision of companies working within the protection zone of main pipeline (MP) or at the emergency area;

  - Quality control and verification of documents prescribed by legal acts;

  - Introductory briefing for Contractor’s personnel, issuing of passes/permits;

  - Required sampling of soil and water, analyzing of the samples for determination of pollution level and treatment progress.

- Materials, Equipment and Services to be provided by the Service Provider.

  - Services:

    - Emergency localization;

    - Installation of soil barriers, dikes, lagoons;

    - Installation of barriers (booms) across rivers, trenches;

    - Erection of sorbent barriers over  rivers, trenches and other water bodies;

    - Collection, shipping out, disposal of used sorbent booms;

    - Installation of channels for directing of the flow of crude or petroleum products to  collection point;

    - Emergency response;

    - Collection of spilled crude oil and/or petroleum products from water surface and contaminated soil (using skimmers, crude oil and petroleum product vacuum pumps, vacuum machines adapted to collect oil products from water surface and other equipment intended for this purpose, including manual tools);

    - Discharge, collection, shipping out and disposal of absorbents;

    - Removal of spilled crude oil or petroleum products, also of contaminated water/soil for recovery and/or recovery thereof. The Contractor shall ship spilled crude oil or petroleum products and contaminated water/soil to Public Company ORLEN Lietuva crude oil refinery waste recovery/disposal facilities at Juodeikiai Vill., Mažeikiai Distr., or the Contractor shall manage (treat) such waste itself if agreed so with the Owner. The Contractor shall be required to have the technical capabilities and licenses (carriage of dangerous goods and management of hazardous waste) required for the shipment and/or management of spilled crude oil or petroleum products and contaminated water/soil;

    - Treatment of soil and water areas contaminated with crude/petroleum products with chemical and microbiological preparations approved by state environmental authorities (approval for use of chemical and microbiological preparations shall be obtained by the Contractor);

    - Delivery of soil, sand or gravel to the area of an emergency;

    - Cleaning of an emergency area, leveling of dikes, restoring of the banks of ditches/channels, delivering of soil to replace the removed contaminated soil, leveling of the land surface, and sowing of grass in the area of an emergency;

    - Collecting of crude spills on the seaside, spreading and collecting of absorbents, removing of the absorbents and contaminated sand to recovery sites, delivering of clean sand for replacement of the removed contaminated sand;

    - Other works specified by the Owner (including preparation of an occupational safety procedure for the works to be performed).

  - The Contractor shall provide the following machinery, equipment and tools necessary for works, also lighting:

    - Cranes, forklifts, tractors, excavators, bulldozers, self-propelled hydraulic impact hammers;

    - Trucks, haulage trucks;

    - Equipment for pollutant collection from water – skimmers, liquid pollutant collection from ground surface equipment, in coastal zone – vacuum collectors, vacuum tanks, tank trucks, liquid spraying units;

    - Power generators, air compressors, lighting equipment, etc. (where required).

  - Materials required for the work:

    - Floating booms, absorbent booms, sorbent booms, tanks – inflatable, folding with buoys, etc.;

    - Peat absorbent, polypropylene absorbent, sorbent pillows, etc.;

    - Dispersants, chemical, microbiological treatment preparations;

    - Soil, sand, gravel, etc.

  - The Contractor shall provide its employees with all occupational health and safety equipment, work clothes in line with the Owner approved OHS Program for Contractors BDS-40.

  - The Contractor shall provide fuel and full maintenance for its equipment and/or vehicles.

  - The Contractor shall provide transportation for its staff to/from the work site.

  - The Contractor shall arrange transportation of materials, tools, machinery to the site of works, and have vehicles required for this purpose.

- NOTE:

- If the Contractor transports spilled crude oil and/or petroleum products and contaminated water/soil for handling (disposal) to the waste disposal/recovery facility of crude oil refinery of AB ORLEN Lietuva in Juodeikiai Vill., Mažeikiai Distr., the Owner via the Unified Product, Packaging and Waste Record Keeping Information System (GPAIS) will issue a waste accompanying document.

- If spilled crude oil and/or petroleum products and contaminated water/soil is transferred for disposal/recovery by the Contractor, before shipping waste out of the area of an emergency, the Contractor will be required to enter the contract details into GPAIS, and the Owner will issue waste accompanying document via GPAIS.

- Hazardous waste must be managed in accordance with the requirements prescribed by the General Terms & Conditions of Waste Management Contract.

- Level I works shall be deemed completed when:

  - Such are not continued under Level II;

  - The consequences of an emergency (pollution) have been eliminated and this is proved by the findings of soil and/or water control tests, and respective conclusion of environmental authority has been received;

  - The territory of work is orderly, waste has been disposed of;

  - Full reporting documentation (including soft copies) specified in the contract has been presented.

  - If works are continued under Level II,

  - the works shall be documented according to actual scope of completed works agreed with the Owner.

- Work schedule:

  - The Contractor must inform the Dispatchers of Main Pipelines Service Group of Pipeline and Terminal Operations Subdivision (Biržai, tel.: +370 443 93483; Būtingė, tel.: +370 443 93459) and the Work Supervisor appointed by the Owner on the time of start and end of work on each workday.

  - The Contractor must start, execute and finish all works according to the terms prescribed by Order(s), legal acts [5.4, 5.5, 5.6, 5.8] and mandatory ordinances of environmental authority.

  - The Contractor shall plan for and have a sufficient number of employees to perform works in a timely and quality manner.

  - Before starting works, the Contractor shall provide mobilization schedule of required machines, equipment, materials and personnel to the below objects for the Owner’s approval:

    - Biržai Oil Transshipment Station, Beržyno Str. 7, Stačkūnai, Biržai Distr.;

    - Rokiškis, Rokiškis Municipality (Obeliai Vill., Juodupė Vill.);

    - Būtingė Terminal, Terminalo Kelias-2, Palanga;

    - Mažeikių Str. 75, Juodeikiai Vill., Mažeikiai Distr.;

    - Joniškis Oil Transshipment  Station, Valiūnai Vill., Joniškis Distr.

  - Time shall be calculated from the receipt of notification from the Owner until the arrival at the object specified by the Owner.