- General data:

Main crude oil pipeline Mažeikiai – Būtingė:

Pipeline diameter – 22 inches;

Length in the territory of the Republic of Lithuania – 92 km;

Transported product – crude oil;

Operation pressure – 100 bar.

Offshore pipeline, on-shore section:

Pipeline diameter – 36 inches;

Length in the territory of the Republic of Lithuania – 3 km;

Transported product – crude oil;

Operation pressure – 12 bar.

Main product pipeline Ilukste – Biržai – Ventspils:

Pipeline diameter – 530 mm;

Length in the territory of the Republic of Lithuania – 87 km;

Transported product – diesel fuel;

The pipeline stands idle, product pumped out. Filled with nitrogen gas, pressure – up to 3 bar. Low danger to environment.

Main crude oil pipeline Polotsk – Biržai – Ventspils:

Pipeline diameter – 720 mm;

Length in the territory of the Republic of Lithuania – 87.384 km;

Transported product – crude oil, operation pressure – 39 bar.

The pipeline stands idle, crude oil pumped out, pipeline washed and mothballed. Filled with nitrogen gas, pressure – up to 3 bar. No potential danger to environment.

Main crude oil pipeline Polotsk – Biržai – Mažeikiai:

Pipeline diameter – 720 mm;

Length in the territory of the Republic of Lithuania – 225.514 km;

Transported product – crude oil, operation pressure – 51 bar.

The pipeline stands idle, crude oil pumped out, pipeline washed and mothballed. Filled with nitrogen gas, pressure – up to 3 bar. No potential danger to environment.

- Attachments

Attachment 1. Quantities and delivery terms of obligatory emergency response means and qualified staff.

Attachment 2. Unit rates and hourly rates for emergency response means, services and qualified staff.

- Description and Specifics of Services.

Remediation of land contaminated with hydrocarbons within the territories of AB ORLEN Lietuva operated main crude oil pipelines Polotsk – Biržai – Ventspils, Polotsk – Biržai – Mažeikiai, Mažeikiai – Būtingė, main product pipeline Ilukste – Biržai – Ventspils, on-shore section of marine pipeline (from Terminal to the sea).

  - Normative documents, standards and other regulations to be observed by the Contractor for containment of and response to emergencies:

    - Rules for Operation of Main and Offshore Crude and Petroleum Product Pipelines approved by the Minister of Economy of the Republic of Lithuania with Order No. 4-207 dated 25 May 2007;

    - Rules for Protection of Main Crude Oil Pipelines approved by the Minister of Economy of the Republic of Lithuania with Order No. 1-131 dated 30 April 2010;

    - Law on Environmental Protection of the Republic of Lithuania (21 January 1992 No I-2223 (as amended));

    - 17 November 2009 Order No D1-694 by the Minister of Environment of the Republic of Lithuania regarding approval of Lithuanian environmental normative document LAND 9-2009 - Environmental Requirements for Remediation of Territories Contaminated with Petroleum Products;

    - 16 May 2006 Order No D1-228 by the Minister of Environment of the Republic of Lithuania regarding approval of the Procedure for Selection of and Preliminary Approval of Remedial Measures (Official Gazette, 2006, No 59-2099, as amended);

    - 30 April 2008 Order No D1-230 by the Minister of Environment of the Republic of Lithuania regarding approval of environmental requirements for remediation of territories contaminated with chemical substances (as amended);

    - 17 June 2008 Order No 1-104 by Director of Lithuanian Geology Survey under the Ministry of Environment regarding approval of Eco-Geological Investigation Regulations (Official Gazette, 2008, No 71-2759, as amended);

    - 9 September 2002 Order No 471 by the Minister of Environment of the Republic of Lithuania regarding approval of Methodology for Calculating Environmental Damage Compensation (Official Gazette, 2002 No 93-4026, as amended);

    - If the Contractor is going to perform waste management operations, it must meet all the requirements applicable to waste management companies prescribed by the Law on Waste Management of the Republic of Lithuania (Official Gazette, 1998, No 61-1726, as amended) and observe Waste Management Rules approved by the Minister of Environment with the 14 July 1999 Order No 217 (Official Gazette, 1999, No 63-2065, as amended), and the requirements established in Attachment 5.

    - In case of shipment of waste from LR to EU, the Contractor shall comply with the requirements of transboundary movement of waste, as established in Regulation (EC) No 1013/2006 of the European Parliament and of the Council of 14 June 2006 on shipments of waste, and comply with  Order No D1-384 of 4 July 2007 by the Minister of Environment regarding amendment of Order No D1-207 as of 27 April 2004 by the Minister of Environment of the Republic of Lithuania regarding approval of Waste Shipment Procedure (Official Gazette, 2007, No 76-3036).

NOTE: The Contractor shall confirm that it is familiar with the above-mentioned legislation and other applicable standards and requirements.

- Materials, equipment, and services to be provided by the Owner.

  - Marking of underground utilities (cables, piping);

  - Supervision of companies operating within the protection zone of main pipeline (MP) or at the emergency area;

  - Quality control and verification of documents prescribed by legal acts;

  - Introductory briefing for Contractor’s personnel, issuing of passes/permits.

- Materials, Equipment and Services to be provided by the Service Provider.

  - The Contractor, using its own resources (provided that it holds the license required for the particular kind of works) or subcontractors, in compliance with the requirements and deadlines prescribed by the applicable legal acts (3.3.4, 3.3.5, 3.3.6), shall:

    - Perform preliminary and, if required, detailed environmental surveying of a contaminated area, present survey reports to the Owner and agree them with environmental authorities and Lithuanian Geology Survey (LGS);

    - Based on the findings of preliminary and/or detailed environmental surveying of a contaminated area, no later than within 7 days after completion of emergency response operations, prepare the Plan of Remedial Measures and agree it with the Owner and environmental authorities.

  - Works to be performed by the Contractor according to the agreed Plan of Remedial Measures:

    - Territory cleaning/recovery using methods allowed within the Republic of Lithuania – chemical and biological preparations. The preparations shall have respective licenses, safety data sheets and user manuals. Soil cultivation during the cleaning/treatment of the territory;

    - Soil, water sampling to identify the cleaning progress, erection of observation and pollutant collection wells (as required);

    - Preparedness for pollution containment and elimination of dispersed pollution (during melting, excess rainfall periods);

    - Arrangement of absorbent booms, spreading, collection of absorbents and transfer thereof to waste handling entity or handling thereof (where licenses required for this purpose are held);

    - As needed, transportation of spilled crude oil and/or petroleum products and contaminated water/soil for handling thereof to the waste disposal/recovery facility of crude oil refinery of AB ORLEN Lietuva in Juodeikiai Vill., Mažeikiai Distr.;

    - As needed, upon agreement with the Owner, handling of spilled crude oil and/or petroleum products and contaminated water/soil at the Contractor's waste disposal/recovery facility, provided that the Contractor has technical capabilities and licenses (carriage of dangerous goods and waste management) required for this purpose;

    - Collection and laboratory analysis of control samples to verify the efficiency of remediation of contaminated territory;

    - Other works under the Plan of Remedial Measures.

  - Mobilization/demobilization of the Contractor's staff and equipment to the location specified by the Owner.

  - After completion of remediation of a contaminated territory, a final report shall be issued which, once agreed with the Owner, shall be presented to LGS. (The final report on territory remediation shall be presented to LGS in hard and soft copy.)

  - If works in a contaminated territory persist for more than 6 months, interim reports shall be prepared every half-year, and an updated Contaminated Territory Remediation Plan shall be agreed with environmental authorities.

  - Provision of machinery, tools and equipment necessary for works is a responsibility of the Contractor.

  - Provision of all materials required for work is a responsibility of the Contractor.

  - The Contractor shall provide its employees with all occupational health and safety equipment, work clothes in line with the OHS Program for Contractors BDS-40.

  - The Contractor shall provide fuel and full maintenance for its equipment and/or vehicles.

- NOTE:

- If the Contractor transports spilled crude oil and/or petroleum products and contaminated water/soil for handling (disposal) to the waste disposal/recovery facility of crude oil refinery of AB ORLEN Lietuva in Juodeikiai Vill., Mažeikiai Distr., the Owner via the Unified Product, Packaging and Waste Record Keeping Information System (GPAIS) will issue a waste accompanying document; if spilled crude oil and/or petroleum products and contaminated water/soil is transferred for disposal/recovery by the Contractor, before shipping waste out of the area of an emergency, the Contractor will be required to enter the contract details into GPAIS, and the Owner will issue waste accompanying document via GPAIS.

- Hazardous waste must be managed in accordance with the requirements prescribed by the General Terms & Conditions of Waste Management Contract. The Owner reserves the right to outsource a different contractor for works specified in sub-items of Par. 5.1.

- Works shall be considered fully completed if:

  - Works specified in the Plan of Remedial Measures have been completed in full; final remediation report has been prepared as prescribed by legal acts, agreed with the Owner and transferred to LGS for review;

  - Environmental authorities, based on the conclusions from LGS, have decided that the territory remediation targets have been achieved and no additional remedial measures are required;

  - The site of works has been tidied up; waste and any irrelevant objects (e.g., booms, temporary waste containers, siphons, etc.) have been collected and removed.

- Work schedule

  - The Contractor must inform the Dispatchers of Main Pipelines Service Group of Pipeline and Terminal Operations Subdivision (Biržai, tel.: +370 443 93483; Būtingė, tel.: +370 443 93459) and the Work Supervisor appointed by the Owner on the time of start and end of work on each workday.

  - The Contractor must start, execute and finish all works according to the terms prescribed by the Contaminated Territory Remediation Plan and applicable legal acts.

  - The Contractor shall plan for and have a sufficient number of employees to perform works in a timely and quality manner.

  - Upon receipt of an order, the Contractor shall arrive at the location indicated by the Owner for agreement on the works to be done as soon as possible, but not later than within 24 hours.