Requirements for computers and software

- The latest (up-to-date) operating system (Windows) available at the time of system installation.

Computer manufacturer – Dell, HP. Warranty – 3 years. Minimum requirements for computers:

- Computers shall support Windows 11;

- Active operating system OEM licenses;

- Not more than 60% system usage (RAM, CPU, SSD) after installation of all required programs;

- Hard disc for workstations, servers – SSD, RAID-1, hard disc for data storage – SSD, RAID-5;

- RAM 32GB.

- Network cards – at least 3.

- Screen – AntiGlare, 23-24“. Arm brackets with C clamps for monitor mount.

- Audio speakers can be integrated into the operator workstation monitors.

- Remote monitor/keyboard/mouse/audio (KVM) connection to operator and engineer workstations – CATx.

- The Contractor shall provide antivirus software. License – for 1 year.

- System operator workstation shall be connected to computer using a KVM extender.   Distance – approximately 50m.

- Contractor shall provide software (preferably – Veritas) for online backup copies.

- The Contractor shall provide copies of the software installed and licenses for such copies.

Requirements for programming:

- The Contractor shall follow the best practice when performing programming, add additional explanations, comments, etc. The programming shall not use locked/inaccessible logic blocks. The Owner shall be able to make changes to the program if necessary without contacting the Contractor.

- If licensed or proprietary program blocks or functions are used, an explanation of how the blocks or functions work (without infringing the copyright) shall be included in the programming package or in a separate document.

- The system shall have a diagnostic window(s) to show the status of processors, modules, communications, etc., and generate appropriate fault alarms.