Scope of Work

Inspection, repair, and testing of TPP-64 6 kV switchgear sections 2, 3, 4, 5, and TPP-65 6 kV switchgear sections 1, 2, 3

1. General data.

Technical maintenance and repair of 6 kV electrical equipment, GP-2 TPP-64 , TPP-65.

2. Attachments.

- TPP-64 / TPP-65 US-6 kV repair description

- TPP64_US6kV diagram

- TPP65_US6kV diagram

- Requirements for electrical work certificates issued by VERT

- 2027_Outage Schedule

3. Description and Specifics of the Work

3.1. Work Description

TPP-64 – 6 kV Switchgear, Section 2

- Overhaul of TPP-64 6 kV Section 2 switchgear (details provided in the attachment).

- Repair of circuit breakers (6 units, the number may increase after the initial inspection), VT and CT repair (2 units) (details in attachment).

- Repair of busbars and insulators in 13 cubicles (details in attachment).

- Testing of circuit breakers (10 units) according to testing norms and scope.

- Measurement/testing of VTs and CTs (2 units) according to testing norms and scope.

- Testing of power cables (3 units): TP-77/T2; TPP-63 KTP-1/T1; TPP-63 KTP-2/T2

TPP-64 – 6 kV Switchgear, Section 3

- Overhaul of TPP-64 6 kV Section 3 switchgear (details in attachment).

- Repair of circuit breakers (6 units, may increase after inspection), VT and CT repair (2 units).

- Repair of busbars and insulators in 12 cubicles.

- Testing of circuit breakers (12 units).

- Measurement/testing of VTs and CTs (2 units).

- Testing of power cables (4 units): TPP-64 KTP-1/T2; TPP-64 KTP-2/T1; TPP-64/3; KTP-1/T1

TPP-64 – 6 kV Switchgear, Section 4

- Overhaul of TPP-64 6 kV Section 4 switchgear.

- Repair of circuit breakers (3 units, may increase after inspection) and VT repair (2 units).

- Repair of busbars and insulators in 6 cubicles.

- Testing of circuit breakers (6 units).

- Measurement/testing of VTs (2 units).

- Testing of power cables (1 unit): KTP-2/T1

TPP-64 – 6 kV Switchgear, Section 5

- Overhaul of TPP-64 6 kV Section 5 switchgear.

- Repair of circuit breakers (5 units, may increase after inspection) and VT repair (2 units).

- Repair of busbars and insulators in 6 cubicles.

- Testing of circuit breakers (5 units).

- Measurement/testing of VTs (2 units).

- Testing of power cables (2 units): TPP-64/2; KTP-2/T2

TPP-65 – 6 kV Switchgear, Section 1

- Overhaul of TPP-65 6 kV Section 1 switchgear.

- Repair of circuit breakers (2 units, may increase after inspection), VT and CT repair (2 units).

- Repair of busbars and insulators in 14 cubicles.

- Testing of circuit breakers (10 units).

- Measurement/testing of VTs and CTs (2 units).

- Testing of power cables (1 unit): TPP-65 KTP-1/T2

TPP-65 – 6 kV Switchgear, Section 2

- Overhaul of TPP-65 6 kV Section 2 switchgear.

- Repair of circuit breakers (4 units, may increase after inspection), VT and CT repair (2 units).

- Repair of busbars and insulators in 14 cubicles.

- Testing of circuit breakers (12 units).

- Measurement/testing of VTs and CTs (2 units).

- Testing of power cables (2 units): KTP-1/T1; KTP-2/T2

TPP-65 – 6 kV Switchgear, Section 3

- Overhaul of TPP-65 6 kV Section 3 switchgear.

- Repair of circuit breakers (3 units, may increase after inspection), VT and CT repair (2 units).

- Repair of busbars and insulators in 12 cubicles.

- Testing of circuit breakers (11 units).

- Measurement/testing of VTs and CTs (2 units).

- Testing of power cables (1 unit): TPP-65/3; KTP-2/T1

3.2. Specific Requirements (if applicable)

There are no specific requirements.

3.3. Requirements for Work Execution and Documentation

After completion of the repair works, the contractor must carry out all necessary measurements and tests specified in the “Electrical Equipment Testing Norms and Scope” and the “Electrical Installation Regulations.”
All test reports, measurement protocols, and other relevant technical documentation must be submitted upon completion.

3.4. Contractor Qualification Requirements

The Contractor and Subcontractors must hold valid certificates authorizing them to perform the following activities:

Certificate issued by the State Energy Inspectorate of the Republic of Lithuania, granting the right to operate electrical equipment:

- Operation of electrical installations up to 10,000 V;

- Operation of electrical networks and equipment up to 10,000 V;

- Testing of electrical networks and equipment up to 10,000 V.

A complete list of required certificates is provided in the attachments.

4. Materials, Equipment, and Services Supplied by the Client

If required by the contractor for the performance of the repair works, the Client will supply all materials related to the repair, except for lubrication and cleaning agents used during the repair process.

5. Materials, Equipment, and Services Supplied by the Contractor

The Contractor must provide and include in the price all materials, equipment, and services required for the performance of the works, except for lubrication and cleaning agents, which are not supplied by the Client.

6. Requirements for Work Completion

Define the key stages, deadlines and control points during the execution of works when the Client will review, approve, or reject the completed work.

The Contractor must:

- Handover–Acceptance Statement for Performed Works

- Submit repair, testing, and electrical measurement protocols.

7. Requirements for Work Acceptance

The works are considered fully completed when, in accordance with the procedure established by AB "ORLEN Lietuva", the Contractor submits:

- repair protocols,

- testing protocols,

- electrical measurement reports

- Handover–Acceptance Statement for Performed Works

8. Requirements for the Work Schedule

The period is planned to coincide with the ORLEN Lietuva Refinery TA2027 (preliminarily scheduled for spring 2027; the exact start date and duration will be specified later).