1. General

AB ORLEN Lietuva (the OWNER) plans to install isolation valves with fire protection in the bottom of tower K-201 (Operations Subdivision No 2 section S-200) according to HAZOP.

2. Attachments

2.1. Design OLP02585: installation of isolation valves with fire protection in the bottom of tower K-201 at Operations Subdivision No 2 section S-200 - Process Control and Automation (PCA) Design with attachments.

3. Scope of Work and Work Specifics

3.1. General requirements

3.1.1. CONTRACTOR shall visit the site for full evaluation of the existing infrastructure and the scope of work. All costs associated with the preparation of the proposal shall be exclusively at CONTRACTOR'S expense.

3.1.2. Before submitting a proposal, the CONTRACTOR shall analyze this Scope of Work and ensure that all tasks described in this document and its attachments are incorporated into the proposal. Any minor tasks not explicitly mentioned in this Scope of Work, but required to ensure full functionality and completeness of works, shall be considered included in the proposal.

3.1.3. The CONTRACTOR shall supply all equipment, manpower and services, provide installation management services, work supervision, overhead costs and any other services related hereto. In the event of conflicts between documents, or if the technical information provided in the SOW or its attachments is incomplete, the CONTRACTOR shall promptly inform the OWNER’S Project Manager or their authorized representative and act in accordance with their instructions.

3.1.4. CONTRACTOR shall perform works according to the standards, normative and legal acts effective in the Republic of Lithuania, AB ORLEN Lietuva specifications and requirements.

3.1.5. The CONTRACTOR shall obtain all permits and authorizations for entry/exit of its equipment and work tools into/from the territory of AB ORLEN Lietuva in accordance with AB ORLEN Lietuva requirements.

3.1.6. At the end of each shift, CONTRACTOR shall clean up the work site, sort out and dispose of the waste generated during the works. Waste shall be removed by the CONTRACTOR using its own transport to the area located in the territory of the OWNER. AB ORLEN Lietuva Rules on Waste Management Activities shall be observed.

3.1.7. All works shall be performed in accordance with AB ORLEN Lietuva occupational health and safety procedures.

3.1.8. Where necessary, CONTRACTOR shall provide its staff with offices and sanitary facilities and shall take care of the premises by keeping them clean and in good order.

3.2. Work Description

3.2.1. The CONTRACTOR shall perform all installation works required in the process control and automation part.

3.2.2. The CONTRACTOR shall procure all materials, instruments, DCS equipment etc. required for the design (including minor materials not foreseen in the design).

3.2.3. The CONTRACTOR shall perform measurements, tests, loop checks, as required by the Electrical Equipment Installation Rules and other regulations.

3.2.4. The CONTRACTOR shall prepare and compile as-build design documentation, various reports/statements and transfer them to OL archive.

3.2.5. The CONTRACTOR shall perform troubleshooting during the commissioning of equipment.

3.3. Special requirements (if any)

3.3.1. The CONTRACTOR must be experienced in performing this type of work.

3.3.2. The CONTRACTOR shall clean the worksite at the end of each working day, remove the waste resulting from the works to the area indicated by the Owner.

3.3.3. The CONTRACTOR shall, at its own expense, eliminate any damage caused to instruments and any other equipment during installation and commissioning.

3.3.4. The CONTRACTOR, together with the Owner, shall sign a Statement on Occupational Safety and Health Mutual Responsibility Limits.

3.3.5. The CONTRACTOR shall comply with Public Company ORLEN Lietuva work rules and regulations.

3.3.6. Components, equipment and materials supplied by the CONTRACTOR shall comply with the CE standards, have CE certificates and shall be certified as required by ATEX Directive.

3.4. Requirements for work execution and documentation

The CONTRACTOR shall submit the planned working hours, work schedule (taking into consideration that the installation works will be executed in stages, with the operating unit, and without interruption of technological process) and separate cost estimates for works and materials. The CONTRACTOR shall evaluate possible contingent work.

3.5. Qualification requirements for Contractor

The CONTRACTOR shall provide proof of qualification or submit a written confirmation that the required documents will be presented before the conclusion of the contract:

3.5.1. Valid certificate issued by National Energy Regulatory Council of the Republic of Lithuania (Lith. VERT) for operation of energy facilities (electrical, thermal, liquefied petroleum gas, crude oil and petroleum products, natural gas), authorizing the performance of the works specified in the SOW, i.e., for certain types of works and activities.

3.5.2. The CONTRACTOR shall present a list of subcontractors and proof of their qualification (if applicable).

4. Materials, equipment, and services to be provided by the Owner

4.1. Scaffolds;

4.2. Insulation works;

4.3. Electric actuator Rotork IQ3 with PST function - 3 pcs;

4.4.  DI 32-channel module (24VDC dry contact) -1 pc;

4.5.  DO 32-channel module, completed with 4 DO plates - 1 pc;

4.6. DO module and plate junction cable - 2 pcs;

4.7. Module plate -1 pc;

4.8. Terminal block for wires up to 2.5 mm², spring‑loaded wire clamps – 42 pcs;

4.9. DIN rail end stop - 6 pcs;

4.10. Auxiliary contacts 2NO block, 230V, 3A - 3 pcs;

4.11. Control station -3 pcs;

4.12. Copper‑core control cable, operating voltage 300 V, fire-retardant PVC insulation, UV resistant - 45 m;

4.13. Copper‑core fire-resistant control cable, operating voltage 0.6/1kV, orange 14G 1.5 -1000 m;

4.14. Copper‑core fire-resistant control cable operating voltage 0.6/1kV, orange 7G 1.5 -15 m;

4.15. Installation wire, gray 0.75 mm2 - 20 m.

5. Materials, equipment and services to be provided by the Contractor.

5.1. The CONTRACTOR shall supply all materials (except the ones indicated in Par. 4) required by the design (including minor ones not included in the design).

5.2. The CONTRACTOR shall provide services and furnish equipment, safety devices and transport required for the works.

5.3. Materials furnished by the CONTRACTOR and required for the design project must be new, certified and of high quality.  Their manufacturers must be selected from the list of suppliers acceptable to the OWNER, whereas any changes must be pre-agreed with and meet the OWNER’s requirements.

5.4. Materials shall be supplied together with TECHNICAL DOCUMENTATION as required by the regulatory documents of the Republic of Lithuania and by the Owner (declarations of conformity, certificates, manufacturer test reports, operation manuals, operational maintenance procedures, etc. in Lithuanian).

6. Requirements for work completion

6.1. The CONTRACTOR has completed all construction and installation works, measurements and tests, commissioning and start-up activities, and has prepared a MECHANICAL COMPLETION ACCEPTANCE STATEMENT, which has been signed by the OWNER.

6.2. Following the OWNER'S procedure, the CONTRACTOR has prepared and submitted to the OWNER the technical documentation of the project, also remedied any identified defects and nonconformities.

6.3. Obtain a permit for operation from the Technical Inspection and Analysis Group of the Electrical and Automation Department.

6.4. The CONTRACTOR must clean up its workplace/site, sort out and remove waste.

7. Requirements for work acceptance

7.1. The CONTRACTOR must present a full set of AS BUILT design documentation in electronic format for transfer to archive.

7.2. The parties (Contractor and Owner) sign a final work handover and acceptance statement.

8. Requirements for work schedule

Installation, commissioning/startup works are scheduled for Turnaround 2027 (exact date will be provided later). The proposal shall include a work execution schedule. Material delivery schedule.

Prepared by:  Instrumentation Engineer Marius Ivanauskas, tel.: +370 649 66001