|  | PUMP MECHANICAL SEAL SELECTION SHEET |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Client |  |  |  |  |  |  |  |  |  |  |
|  | Company name, location: |  |  |  | Orlen Lietuva GP-2 S-200 |  |  |  |  |  |  |
|  | Responsible person name, position, contact information: |  |  |  | Saulius Gestautas Senior Engineer tel. 37069819593 . e-mail saulius.gestautas@orlenlietuva.lt |  |  |  |  |  |  |
|  | Equipment information |  |  |  |  |  |  |  |  |  |  |
|  | Pump manufacturer, model: |  | Volgogrado gamykla TKA-63/125 |  |  |  | Horizontal Vertical |  | Console unit Horizontal split |  |  |
|  | Pump position tag: |  | S-211/R |  |  |  |  |  |  |  |  |
|  | Speed RPM: |  | 2950 |  |  |  | Rotation direction CW/CCW: |  | ccw |  |  |
|  | Pressure in seal chamber (barg): |  |  |  | 2 barg |  | Seal chamber cooled: |  | Yes No |  |  |
|  | Pump location: |  |  | Indoors Outdoors |  |  | Known pump problems specific to seal: | GOST standarto sandariklio keitimas |  |  |  |
|  | Pumped media information |  |  |  |  |  |  |  |  |  |  |
|  | Product name (composition, formula, concentration): |  |  | Condensate (condensate cosists of water and ammonia(<10g/l) with impurities of H2S(<5g/l)) |  |  |  |  |  |  |  |
|  | Working temperature, °С: |  |  | 109 °C (100 – 110 °С) |  |  | Density (g/cm3): | 0,937 |  |  |  |
|  | Viscosity at ambient Т° (cct or mPa s): |  |  | 0,90 |  |  | Viscosity at working Т° (cct or mPa s): | 0,24 |  |  |  |
|  | Solids, % volume capacity and dimensions, mm: |  |  | None |  |  | Known media features (toxic, flammable, polymerising etc.) | Suffocating odor, corrosive |  |  |  |
|  | Seal design preferences |  |  |  |  |  |  |  |  |  |  |
|  | Seal design standard |  |  | API 682 3th edition |  |  | Seal materials preferences |  |  |  |  |
|  | Seal type, arrangement preferences |  |  | Double seal Category 2 |  |  | Piping plan preferences | 53B 53A |  |  |  |
|  | Note : Mechanical seal manufacture : Burgmann; John Crane; Flowserve Corporation, KSB, CHETRA Dichtungstechnik AG, AESSEAL . Provide the seal drawing with indicated dimensions, as well as installation and operating instructions. With thermosiphon system for API plan 53B with pressuretransmitter (4-20mA) HART and temperature Indicator. Thermosiphon system mounted on a stand. With thermosiphon system for API plan 53A, with pressure transmitter (4-20mA) HART, IP > 65, explosion protection Ex ia, level transmitter (4-20mA) HART, IP > 65, explosion protection Ex ia. Should be provided with calibration certificate, ATEX certificate and Material 3.1 certificate. Pressure gauge dial size ø100 mm. with laminated safety glass, stainless steel 316L, liquid filled case, bottom connection, process connection M20x1,5, IP65. Thermosiphon system mounted on a stand... |  |
|  | Installation dimensions and limits |  |  |  |  |  |  |  |  |  |  |
|  | Ds | Shaft diameter, mm |  |  |  |  |  |  |  | Ø60 h6 |  |
|  | Ds1 | Shaft sleeve diameter if present, mm |  |  |  |  |  |  |  | no |  |
|  | Dk | Seal chamber diameter, mm |  |  |  |  |  |  |  | Ø110 H9 |  |
|  | Lk | Seal chamber length (depth), mm |  |  |  |  |  |  |  | 115 |  |
|  | Ls1 | Shaft sleeve length if present (distance from pump bore to sleeve end), mm |  |  |  |  |  |  |  | no |  |
|  | Lq | Distance to pump connections (if used), mm |  |  |  |  |  |  |  | no |  |
|  | Lo | Distance to nearest axial obstruction, mm |  |  |  |  |  |  |  | 126 |  |
|  | Do | Distance to nearest radial obstruction, mm |  |  |  |  |  |  |  | 200 |  |
|  | Lb | Bolts size and quantity |  |  |  |  |  |  |  | M16 4pcs |  |
|  | Db | Bolt circle diameter, mm |  |  |  |  |  |  |  | 165 |  |
|  | a | Bolt location angle to horizontal axis, |  |  |  |  |  |  |  | 30 |  |