Appendix 1 to the Request for Proposal

Terms of Reference of the Cabin Patrol and Rescue Boat for ORLEN Ochrona Branch in Gdańsk

Intended Use:

The patrol and rescue boat will be used to patrol and protect the security zone of the ORLEN S.A. Marine Fuel Terminal located in the Port of Gdańsk on the Martwa Wisła River throughout the year (except for icing of the water area), marine critical infrastructure facilities of the ORLEN Group in the Baltic Sea and to provide protection services for terminals, protected areas and ships from the water side. The boat will be used on inland and offshore waters, up to and including sea state 4, at air temperatures between -10°C and +35°C.

- Basic Data:

- A boat with complete equipment, manufactured between 2022 and 2025 (boats manufactured before 2022 are not allowed);

- Length of the boat hull: 7.50 meters to 9.50 meters;

- Boat colour:

- Hull, cabin, dashboard and seats – preferred dark colours, not glossy, may be a combination of colours. The superstructure / cabin of the boat in a different colour is allowed. Steel / aluminum elements preferred to be painted in dark colors;

- Hull made of aluminium or polyester-glass laminate in a version adapted to offshore operations. Hull dedicated to planing at high speed (minimum 35 KTS), equipped with water drains from the boat interior. Floor of the boat in non-slip making;

- The boards of the boat made of aluminium or polyester-glass laminate and the stern section protected from external damage in regular daily use.

For RIB boats, an air tube made of Hypalon material, divided into air chambers with two-way valves (pumping / deflating) and relief valves, is acceptable. Rescue rope on the outside of the tube to pick up survivors from the water and inside for grabbing by the boat passengers. A closed-cell foam tube is allowed.

- Cabin – enclosed, for at least four people, providing cover and protection of the crew from cold and flooding by water from the outside, indicated easy access to the cabin, equipped with independent heating, including cold and warm air supply and air vents for ventilation, all windows made of glass, windshields equipped with wipers;

- Seating on the boat for a minimum of six people, including: a helmsman and a second helmsman with shock absorption and backrest, handholds for holding on while surfing on a wave, and for a minimum of two passengers in the cabin (bench or seats) and two people outside in the aft part of the boat (bench or seats)

- Cabin roof with places for mounting antennas and boat lights and additional space for mounting other specialised radio and navigation equipment;

- Rescue equipment in accordance with the requirements for this type of boat, including at least two certified life belts, black inflatable life jackets in accordance with the boat's CE certificate, radar reflector, 6/1 pyrotechnic kit, Epirb 406 MHz radio beacon;

- Bow and stern bollard or dedicated ringbolts to allow safe boat mooring and towing;

- Radio and navigation equipment, at least: marine radar, GPS with plotter (with a screen with a diagonal of at least 10 inches, adapted to install a digital map), echo sounder, stationary VHF radio station with marine channels and DSC function;

- Lighting – in accordance with the requirements for this type of boat; preferred additional boat equipment with a long-range spotlight and working lighting in the cabin, around the outside of the cabin, as well as in the engine room;

- Anchoring kit with chain and rope – as required for this type of boat;

- Fire protection kit – as required for this type of boat;

- Bilge pump set – as required for this type of boat, at least: one electric bilge pump and one manual bilge pump, easy service access to bilge pumps recommended;

- A set of mooring ropes for use with the boat – length: 10 m, minimum thickness: 12 mm, dark colour, 4 pieces.

- Cradle, bed or transport trailer (for servicing or minor repairs of the boat after it has been pulled out of the water).

Comments on point 1:

– Radio and navigation equipment mounted on the boat must have an authorised service in Poland.

- Propulsion, fuel and speed:

- Stationary diesel engine with a minimum power of 250 HP, equipped with a Z-type gearbox with a steel propeller or duoprop with steel propellers,

- outboard gasoline engines, at least two pieces, four-stroke, with steel propellers, with a minimum power of 200 HP each, engine columns adjusted in length according to the boat manufacturer's recommendations.

- The offered boat may have a maximum operating time of the propulsion unit - 300 engine hours, the boat with the smallest number of documented engine hours will be preferred.

- Preferred bow thruster;

- Factory single or double lever in the case of a propulsion unit consisting of two outboard engines, provided by the propulsion unit set manufacturer for this engine set;

- Hydraulic power steering, preferred steering wheel rotation minimum 3 - maximum 6;

- Required clocks: speed indicator, fuel gauge, RPM indicators, battery charging indicators, transmission / engine trim indicators;

- Fuel tank(s) with the required approval for marine operation, with a capacity suitable for the range of the boat of at least 150 nautical miles with a crew of 3 people, at cruising speed of 32 KTS, a boat with a longer range will be preferred;

- Fuel water separators in each engine’s fuel system, with a glass bottom, mounted in a location easily accessible for inspection by the crew;

- The speed of the boat with a full tank of fuel and a crew of 3 people at sea state 1 – at least 35 KTS.

Note: the installed propulsion unit must have an authorised service point in Poland.

- Electrical system:

- All of the boat’s and propulsion unit’s wiring shall be made of cables suitable for marine and salinity conditions;

- Navigation lights and top light, LED technology, 1 set;

- Searchlight – long-range reflector – at least 1 set;

- Engine(s) batteries with mounting kit – 1 piece per engine, designed for marine use; additionally preferred equipment with separate radio-navigational battery

- Boat electric power switches made in a closed version, dedicated to work on boats used at sea, resistant to moist and saline environment.

- Rescue equipment:

- Lifebuoys, 2 pieces, including 1 buoy with a light float attached to the wheel;

- Foam fire extinguishers with mounting in an easily accessible place, with valid approvals for use on the offered boat model, at least 2 pieces;

- Lightweight safety vests operating in dual mode (auto/manual), minimum 6 pieces in black color.

Note: rescue equipment mounted on the boat must have an authorised service in Poland.

- Required documents for bidding in the purchasing procedure:

- a copy of the invoice for the purchase of the boat (with personal information removed – if the purchaser of the boat was a private person);

- a copy of the CE certificate for this boat model;

- a copy of the boat’s registration certificate (only if the boat is or was registered);

- a copy of selected pages of the boat’s guarantee book and the propulsion unit’s guarantee book (applies to pages providing the date of manufacture and date of purchase, full details of the boat and propulsion unit, factory numbers of the boat and engines / transmissions);

- a statement from the seller stating that there have been no accidents involving boats;

- a statement from the seller regarding repairs made to the boat’s hull, cabin and propulsion unit, or information on the lack of repairs to the boat. If repairs were made to the boat, it is necessary to provide in the statement what kind of repairs were made;

- seller’s statement regarding boat insurance policy;

- declaration by the seller regarding the performed service inspections, including the service performing the inspection, the inspection date and the boat's mileage, as well as when the next service inspection is required, for example, after how many engine hours of the propulsion unit and after how much time, if applicable.

- seller’s statement about the data of the offered boat, that is:

- boat manufacturer data;

- boat dimensions;

- data of the boat’s propulsion unit (refers to the specification of the model and power of the engines as well as the model and type of transmission) and the exact number of engine hours worked;

- the boat’s equipment (in separate sections: radio and navigation, electrical, rescue, utilities, additional);

- boat guarantee (including hull / air tube – if applicable) / radio and navigation electronics / propulsion unit.

- Boat category and certification:

- The boat must have a CE certificate – minimum Category C – coastal waters. This boat can sail at wind force up to 6 degrees on the Beaufort scale and waves up to 2.0 meters high;

- The boat must meet the requirements of classification societies for motor boats allowed to navigate coastal and inland waters, in accordance with the requirements for at least category C in accordance with the CE certificate held by the boat.

- Boat delivery and pickup:

- The boat should be delivered in a condition consistent with the offer and the provisions of the sale and purchase agreement to Poland, to the headquarters of ORLEN Ochrona, Branch in Gdańsk, ul. Elbląska 135;

- Delivery time – up to 30 days from signing the contract.

- ORLEN Ochrona will provide a crane for unloading and launching the boat at its working quay.

- Representatives of ORLEN Ochrona and the seller will jointly conduct delivery and acceptance tests of the boat including:

  - verification of the boat’s documentation, the boat’s propulsion unit, and the boat’s complete equipment (as described in the seller’s offer and the sale and purchase agreement);

  - checking the condition of the boat before launching, including the condition of the hull, superstructure and the propulsion unit;

  - checking the nautical characteristics of the boat, including the operation of the steering system of the boat;

  - checking the cruising speed and maximum speed of the boat (according to the offer and provisions of the sale and purchase agreement, with 3 persons on board, full fuel tanks, or more persons and proportionally less fuel), cruise time to check cruising speed – 60 minutes, time to check maximum speed – two attempts, 2 minutes each;

  - checking the condition of the boat, propulsion unit, and boat equipment after delivery and acceptance tests;

  - in the event of positive acceptance of the boat – signing by both parties to the contract of the Delivery and Acceptance Report, which forms the basis for payment of the invoice for the purchase of the boat;

  - during the technical acceptance of the boat, the seller shall be obliged to present a complete set of service manuals for the propulsion system and all electrical equipment installed on the boat, including radio and navigation equipment. All service manuals and boat installation diagrams shall be prepared in either English or Polish.