Scope of Services

It is required to perform the monitoring of diffuse volatile organic compound (VOC) emissions from pollution sources in the refinery of Public Company ORLEN Lietuva. Monitoring must be done using the methods listed in BAT 6 and BAT 18 in accordance with the Commission Implementing Decision 2014/738/EU on best available techniques for the refining of mineral oil and gas.

1. General Requirements

According BAT 6 and BAT 18 to monitor diffuse VOC emissions from the entire refinery by using the following methods:

- Sniffing methods associated with correlation curves for key equipment;

- Optical gas imaging techniques in accordance with known best practice of monitoring;

- Calculations of emissions based on emissions factors periodically validated by measurements.

- Detection method according to EN 15446: 2008, using the hand-held analysers – methodology similar to methodology of Environmental Protection Agency EPA 21 (Method 21 – Determination of Volatile Organic Compound Leaks).

For the purpose of precise description and a clear structure of further content, the following definitions have been introduced:

- point - one potential place of leak,

- element - a device consisting of several potential emission points, e.g. a valve is component that has several potential emission points, i.e. an inlet flange,outlet flange, gland packing, etc.

Optical gas imaging camera will be used during the work as a technique to support rapid leak detection - Smart LDAR methodology.

All potential points of fugitive emission determined by the Purchaser, will be tested using an optical gas imaging camera (OGI).

Quantitative measurements will be perform by applying the detection method according to EN 15446:2008.

Quantitative measurements will be perform in the case when the OGI camera test results will be a leak in the form of "smoke" on the camera screen. In the case of inaccessible elements, where the use of a hand-held analyser will be impossible, monitoring will be limited to the use of the camera. In this case, if a "smoke" is detected, the emission point will be considered leaky without specifying the emission value.

According to the idea of using an optical gas imaging camera as an assistive technique, it is not excluded to conduct monitoring using only a hand-held analyser, without using of the camera.

Such monitoring will be carried out in justified cases, e.g. in situations when meteorological conditions will not allow for proper using of optical gas imaging cameras.

2. Services Description

Monitoring of VOC for measurement points identified by the AB ORLEN Lietuva ( hereinafter,Customer) (valve seals, flanged connections, non-flanged connections, openings in pipelines, sampling points, pump seals, compressor seals, safety valve connections, and other) (from 15000 to 50,000 points per year) and developing a report (3 copies of printed version and an electronic versijon of the report). After completing the works, the Contractor will also provide the Purchaser a set of films confirming the performance of the monitoring.

Detected emission points and leaks must be marked on the installation, and the facility staff must be notified.

Also the number of measured points, identified pollutant leaks locations and concentrations must be sent to the specified e-mail address after each day of measurements.

List of measurement points (from 15000 to 50000 points), diagrams/drawings will be provided by Customer in advance before the agreed date of measurements.

3. Requirements for Documentation

3.1. The report (hard and soft copies) must contain the following information:

- Identification number of pollution source (from the Customer's database);

- Identification number of measurement point (from the Customer's database);

- Type of measurement point (valve seal, flanged connection, etc.);

- Data and time of measurement;

- Measurement results (ppm);

- Method of measurement;

- The Contractor will calculate a value emission of emission in kg/hour using correlations described in

methodology of Environmental Protection Agency EPA 21 (Method 21 – Determination of Volatile

Organic Compound Leaks).

3.2. The report must be issued in English language;

4. Requirements for Service Provider

Monitoring laboratory must be accredited as prescribed by applicable laws for monitoring and testing of emissions, submit a copy of accreditation certificate and accreditation scope.

The service provider must provide a detailed price for measuring of one measuring point upon receipt of an order for measurements in the specified ranges:

- measuring from 15000 to 19999 measurement points,

- measuring from 20000 to 29999 measurement points,

- measuring from 30000 to 39999 measurement points,

- measuring from 40000 to 50000 measurement points.

5. Materials, Equipment and Services to be provided by the Customer:

5.1. The Customer will present a list of points to be measured and diagrams/drawings with marked

measurement points.

5.2.Where necessary the Customer will appoint an accompanying person who will show measurement (monitoring) points.

6. Materials, equipment and services to be provided by Service Provider

6.1. Service Provider must have required monitoring equipment for VOC emissions as required by

BAT 6, BAT 18.

6.2. Monitoring equipment must be Ex.

7. Requirements for Services Completion

Services will be considered completed after the Service Provider submits a report (hard and soft

copies) to the Customer and the Customer signs a services handover-acceptance statement.

8. Requirements for Services Schedule

Customer plans to conclude a contract for a period of 3 (three) years, for the provision of services

from 2027-01-01 to 2029-12-31, respectively, with measurements being taken each year.

Measurements and their corresponding reports shall be completed and submitted respectively by:

2027-12-31 at the latest,

2028-12-31 at the latest,

2029-12-31 at the latest.

9. Occupational health and safety requirements

The Service provider undertakes to familiarize itself and comply with the applicable requirements set

out in the Occupational Health and Safety Procedure for Contractors BDS-40 and in other

occupational health and safety procedures of Customer. The Customer procedures applicable to the

Service provider can be found on website:

https://www.orlenlietuva.lt/EN/ForBusiness/DocumentsForContractors/Pages/Occupational-Safety-and-Health-Documents.aspx