REQUEST FOR QUOTATION

Subject: Development of a Corrosion Control Document (CCD) in accordance with API RP 970

Location: Vinyl Chloride Monomer (VCM) Production Unit P-21, Włocławek

I. SUBJECT, OBJECTIVE AND SCOPE OF THE CONTRACT:

1.1. Subject of the contract:

The subject of the contract is the provision of services involving the development of a comprehensive Corrosion Control Document (CCD) in accordance with API RP 970 for the Vinyl Chloride Monomer (VCM) Production Unit.

1.2. Objective of the contract:

The objective of the project is to prepare complete documentation for identifying, monitoring and mitigating corrosion risk, integrating it with the existing Mechanical Integrity, Management of Change (MOC) and Process Hazards Analysis (PHA) programmes, and using the data in RBI in accordance with API 580/581. The Contractor shall be responsible for:

- Identification of all damage mechanisms in corrosion circuits in accordance with API RP 571;

- Identification of the dominant and secondary damage mechanisms, as well as potential failure mechanisms, for each corrosion circuit;

- Development of Corrosion Materials Diagrams (CMDs) for all process units;

- Definition of Integrity Operating Windows (IOWs) in accordance with API RP 584;

- Identification of Critical Reliability Variables (CRVs) for each corrosion circuit;

- Definition of corrosion risk mitigation strategies and inspection recommendations;

- Integration of the CCD with the existing Mechanical Integrity, Management of Change (MOC) and Process Hazards Analysis (PHA) programmes;

- Development of a data structure enabling use of the CCD in RBI (API 580/581);

- Ensuring unambiguous identification of circuits on P&IDs;

- Integration with CMMS systems;

- Specification of the number and locations of permanent monitoring points and the measurement methods to be used, including the costs associated with their installation.

1.3. Scope of the contract:

- Complete CCD in editable format (Word/Excel) and PDF;

- Corrosion Materials Diagrams (CMDs) for all units;

- Development of a complete list of corrosion circuits;

- Damage-mechanism versus location/material matrix;

- Inspection plan based on identified corrosion mechanisms;

- Gap analysis report against current practices;

- Final presentation for engineering and management personnel;

- Training of personnel in CCD maintenance and updating (minimum 8 hours);

- The Contracting Entity allows for an optional site visit during tender preparation. A site visit is not a prerequisite for participation in the tender procedure (Person responsible for the site visit: Artur Rakoczy, phone: +48 609 160 254).

- ;

- Structured data (Excel/database) covering:

  - circuits,

  - damage mechanisms,

  - IOW/CRV,

  - inspection plan,

  - CCD update procedure (MOC);

  - Governance and maintenance model.

II. TECHNICAL REQUIREMENTS

2.1. Standards and guidelines

The work shall be performed in accordance with the following standards:

- API RP 970 (current edition) - Corrosion Control Document Systems

- API RP 571 - Damage Mechanisms Affecting Fixed Equipment in the Refining Industry

- API RP 584 - Integrity Operating Windows

- API RP 580/581 - Risk-Based Inspection (for reference)

- NACE/AMPP standards applicable to corrosion mechanisms associated with chlorine and chlorine compounds

2.2. Input data to be provided by the Contracting Entity

The Contracting Entity shall provide the following documents:

Process documentation:

- P&IDs (Piping and Instrumentation Diagrams) for all units

- PFD (Process Flow Diagrams)

- Heat and material balances (H&MB)

- Equipment data sheets and piping specifications

- Process descriptions

- Materials of construction specifications

Operating data:

- Operating conditions (temperature, pressure and stream composition)

- Inhibitor and chemical injection points

- Sampling point locations

- Start-up, shutdown and emergency operating procedures

Historical data held by the Contracting Entity:

- Inspection reports from the last five years

- Repair and failure history

- Root cause analyses (RCA) of corrosion-related failures

- Process modification history (MOC)

2.3. Requirements specific to chlorine processes

Due to the aggressive corrosive environment of the VCM Production Unit, the Contractor shall demonstrate knowledge of the following mechanisms:

- Wet chlorine corrosion (>0.5% H₂O)

- Hydrogen chloride (HCl) and hydrochloric acid corrosion

- FeCl3 corrosion in VCM installations

- Corrosion in alkaline environments (NaOH, KOH)

- Stress Corrosion Cracking (SCC) in austenitic steels

- Pitting and crevice corrosion in the presence of chlorides

- Corrosion in EDC (ethylene dichloride) systems

2.4. Corrosion Monitoring System

The Contractor shall develop a monitoring concept covering:

- Selection of techniques (UT, ER probes, coupons, online monitoring);

- Point selection criteria;

- Risk-based prioritisation of circuits and linkage to IOW and RBI.

2.5. Work methodology:

The Contractor shall describe:

  - The methodology for identifying corrosion circuits;

  - The approach to damage-mechanism analysis;

  - The validation process with plant personnel;

  - The documentation quality management system;

  - The communication and meeting plan with the Contracting Entity (minimum three progress review meetings).

2.6. Additional requirements:

- The tender shall be submitted in Polish. An additional English-language version may be attached. Documents shall be provided in PDF and in editable format (Word/Excel).

- All final project deliverables, reports and final presentations shall be provided in Polish. A parallel English-language version may also be provided.

- Integration and maintenance: the CCD shall:

- Support RBI (API 580/581);

- Enable integration with IT systems;

- Include rules for updating and maintenance;

- Be aligned with the MOC process.

- The tender shall include:

- A letter of intent;

- Understanding of the project and its objectives;

- Company background and qualifications;

- Experience in CCD projects (minimum three references);

- Project team composition with CVs;

- Project delivery methodology;

- Detailed schedule;

- Risk management plan;

- Communication and reporting plan;

- Detailed cost calculation;

- Payment schedule linked to milestones;

- Detailed technical and process documentation shall be provided after execution of a Non-Disclosure Agreement (NDA).

III. THE TECHNICAL TENDER SHALL INCLUDE:

- Documents confirming the completion of at least three CCD projects in accordance with API RP 970 for chemical or petrochemical plants, of which at least one project concerned a VCM, EDC or PVC installation, or another chlorine-based process with comparable corrosion hazards.

- Documented experience in corrosion analysis, identification of degradation mechanisms and development of corrosion management strategies for VCM, EDC or PVC installations, or other chlorine-based processes - minimum two references.

- A list of key NACE/AMPP or FROSIO specialists (minimum two persons on the project team), including information on qualifications, certificates and project experience.

- Confirmation that the service will be performed in accordance with the requirements and conditions set out in this document.

- A letter of intent.

- A tender prepared in accordance with the requirements set out in this document.

- Confirmation of readiness to conduct a site visit and technical workshops with representatives of the Contracting Entity during project delivery.

- The Contracting Entity reserves the right to request copies of documents confirming the stated qualifications during tender evaluation or before contract execution.

- Where obtaining a client reference is impossible or difficult, the project was performed on a confidential basis, or the client does not issue formal references, a declaration issued by the Tenderer shall be accepted as sufficient evidence.

The statement should include, among other things:

• Client name (if it may be disclosed);

• Project description;

• Scope;

• Delivery dates;

• Client confirmation of due performance.

IV. SCHEDULE AND PERFORMANCE CONDITIONS

4.1. Completion period

Maximum project completion period: six months from the date of contract execution.

The Tenderer may propose an alternative schedule with justification.

4.2. Milestones

| Phase | Timeline | Deliverables |
| --- | --- | --- |
| Kick-off and data collection | Month 1 | Project plan and document register |
| Unit analysis and corrosion circuit identification | Months 1-2 | Preliminary CMDs and corrosion circuit list |
| Damage mechanism analysis | Months 2-4 | Damage mechanism matrix and IOWs |
| Draft CCD and verification | Months 4-5 | Draft CCD and corrosion monitoring system recommendations |
| Finalisation and training | Month 6 | Final CCD and personnel training |

4.3. Place of performance

The work shall be performed partly remotely and partly at the Contracting Entity's plant in Włocławek. A minimum of three site visits (kick-off, verification and finalisation), each lasting 2-3 days, shall be required.

V. THE COMMERCIAL TENDER SHALL INCLUDE:

5.1. Net price for completion of the entire scope of the contract in accordance with this Request for Quotation.

The price shall include all additional costs necessary for the proper performance of the service.

Please attach a cost estimate containing a detailed calculation covering:

  - Man-hour rates broken down by engineering discipline;

  - Travel and site accommodation costs;

  - Software licence costs (if applicable);

  - Training costs;

  - Consideration of the schedule and milestones.

5.2. Confirmation of the payment term preferred by the Contracting Entity: 30 days from receipt by ANWIL S.A. of a correctly issued invoice, or 60 days from the invoice issue date.

5.3. Advance payments or prepayments shall not be accepted under this contract.

5.4. Proposed settlement method: payments in instalments upon completion of the respective milestones.

5.5. Confirmation of tender validity: minimum 90 days from the tender submission date.

5.6. Confirmation that the price shall remain fixed until completion of the contract.

5.7. The price shall be entered exclusively in the designated field on the CONNECT Platform.

VI. THE FORMAL TENDER SHALL INCLUDE:

1) The proof of company foundation (it is required to enclose the Certificate of Incorporation issued not earlier that 3 months before the deadline for biding in pdf file) ;in the case of consortia, a copy of the consortium agreement and a power of attorney for the consortium leader to represent the other consortium members must be submitted.

2) Confirmation that the Tenderer has read and accepts the following: • Code of Conduct for Suppliers of ANWIL S.A. (document available at https://www.anwil.orlen.pl/PL/StrefaZakupow/Strony/Kodeks-postepowania-dla-Dostawcow.aspx), • provisions of the Anti-Corruption Policy of ANWIL S.A. and Rules for Accepting and Giving Gifts at ANWIL S.A. (documents available at https://www.anwil.orlen.pl/PL/StrefaZakupow/Strony/Wytyczne-ANWIL-dla-Oferentow-i-Wykonawcow.aspx), • provisions of the Human Rights Protection Policy of ORLEN Group (document available at https://anwil.orlen.pl/pl/Oferta/Strefa-zakupow/Wytyczne-dla-oferentow-i-dostawcow).

3) / I declare that I have read and attached a filled Beneficial Owner Declaration document (attached to the RFP)4) A statement acknowledging that the Tenderer has read and accepted the terms and conditions of the document: Conditions for participation in the request for proposal.

5) The Certificate issued by the Tax Office confirming that the company is the active VAT taxpayer issued not earlier than 3 months before the deadline for biding in pdf file.

6) Authorized balance sheet and profit and loss account for the two full reporting years and the most recent - quarterly / half-yearly report for the current period.

7) It is required to enclose VAT declaration for the last 6 months in pdf file.8) The proof that the supplier is not in areas with payment of obligations to the Social Security and the Inland Revenue for the last 6 months (it is required to enclose current certificate in pdf file).

9) Confirmation that, in the event of being awarded the contract, the Contractor shall provide a valid Certificate of Tax Residence.

VII. OWNERSHIP AND RIGHTS

- The Contracting Entity shall acquire full rights to use and modify the developed CCD and the supplied data and materials, within the scope specified in the contract.

The Contractor shall keep confidential all information disclosed by the Contracting Entity and comply with the information protection rules applicable at ANWIL S.A.

IX. ADDITIONAL INFORMATION CONCERNING THE PROCEDURE:

•	All material questions submitted within the specified deadline, together with the answers, shall be published on the CONNECT Platform for all Tenderers participating in the procedure, without identifying the author of the question.

•	Where, in the Tenderer's opinion, a question or answer may result in disclosure of its confidential information, and this fact is expressly reserved in the wording of the question or answer, the response may be provided exclusively to that Tenderer.

•	The Contractor shall comply with all generally applicable site, occupational health and safety, fire protection and environmental protection regulations, as well as the internal regulations of ANWIL S.A. made available to Tenderers.

•	The Contracting Entity reserves the right to conduct multi-stage negotiations in various forms, including direct negotiations and negotiations using an electronic auction system.

•	Tenders obtained during any negotiations using an electronic auction system may be subject to further negotiations with the Tenderers.

•	ANWIL S.A. reserves the right to select the entity or entities with which negotiations will be conducted in relation to part or all of the tender scope, and to enter into a contract with more than one Contractor where justified by the scope of the procedure.

•	ANWIL S.A. reserves the right to discontinue the Contractor selection process and negotiations without stating reasons. Tenderers shall have no claims against the Contracting Entity on this account.

•	This Request for Quotation constitutes an invitation to negotiate within the meaning of Article 72 of the Polish Civil Code.

•	The contract shall be concluded upon signature by ANWIL S.A. and the Contractor. The agreed contractual terms may be confirmed on behalf of ANWIL S.A. only by duly authorised persons.

•	ANWIL S.A. reserves the right to refuse to issue a reference without stating reasons.

X. ATTACHMENTS AVAILABLE ON THE CONNECT PLATFORM:

Enclosure No. 1 - Request for Quotation

Enclosure No. 2 - Service Agreement (One-off Agreement)

Enclosure No. 3 - GTC

Enclosure No. 4 - CONDIDTION FOR PARTICIPATION IN RFP

Enclosure No. 5 - Special Conditions of the Non-Disclosure Agreement

Enclosure No. 6 - Information protection

Enclosure No. 7 - 5.1 OHS

Enclosure No. 8 - 5.2 OHS

Enclosure No. 9 - 5.3 OHS

Enclosure No. 10 - OHS Guidelines for Drivers Entering the Protected Premises of ANWIL S.A.

Enclosure No. 11 - Anti-Corruption Clause

Enclosure No. 12 - Sanctions Clause

Enclosure No. 13 - Declaration - Beneficial OwnerEnclosure No. 14 - Information Clause for Individuals - Contractors or Representatives of ORLEN Group Companies' Contractors - Disclosed to ORLEN as Part of the Centralisation of Trade Credit

Enclosure No. 15 - Information Clause for Individuals - Contractors or Representatives of ORLEN Group Companies' Contractors - Disclosed to ORLEN as Part of the Centralisation of Debt Collection

Enclosure No. 16 - MAR Clause information note

Enclosure No. 17 – Information Clause for a Contractor who is a natural person or conducting business activity, including a partner in a civil law partnership.

Enclosure No. 18 – Information Clause for persons representing the Contractor, designated for contact or cooperating with the Contractor in the conclusion and performance of contracts with ANWIL S.A.

Enclosure No. 20 - Tenderer's Site Visit Statement